CERTIFICATE  OF  ANALYSIS

Product Name: Nicotine

CAS No: 54-11-5

Lot Number：Mj2015-1-6

Quantity：1600KG

Manufacturing Date : .2015

Retest Date：Retest before use

NO:MJ2015-1-8

| Inspection Characteristic | Specification | Test result |
| --- | --- | --- |
| Appearance | Colorless or brownish viscous liquid | Colorless viscous liquid |
| Identification |  |  |
| A.Specific optical Rotation | –140oto –152o | -148 o |
| B.IR: | Comparing with the Ph.Eur.reference Spectrum of nicotine | comforms |
| C.UV: | λmax 257 to 261 nm | λmax = 259nm |
| Appearance of solution: | Clear | Clear |
|  | Not more intense or colour than Y5, BY5 or R5 | Not more than Y5, |
| Water, w/w%: | 0.5 max | 0..16 |
| Residual solvents |  |  |
| Ethanol,w/w%: | 0.5 | 0.041 |
| Hexane,w/w%: | 0.029 | 0.0025 |
| Heavy Metals,ppm: | 10max | Comforms |
| Assay,Non-aqueous titration, w/w%: | 99.0%-101.0% | 99.96 |
| Sulphated ash,w/w %: | 0.1max | 0.01 |
| Related Substances |  |  |
| Nicotine-trans-N-Oxide,area % | 0.2 max | Not detected |
| Nicotine-cis-N-Oxide,area % | 0.2 max | Not detected |
| Myosmine,area%: | 0.5 max | Not detected |
| Continine,area% | 0.2 max | Not detected |
| β-nicotyrine,area% | 0.2 max | Not detected |
| Nornicotine, area% | 0.5 max | Not detected |
| Anabasine.area% | 0.3 max | Not detected |
| Anatabine,area% | 0.3 max | Not detected |
| Any unspecified impurity,area%: | 0.1 max | 0.02 |
| Total Nicotine—N—Oxides, area% | 0.3 max | Not detected |
| Total Impurities(Specified and unspecified), area% | 1.0 max | 0.5 |
| Storage | Under nitrogen, in an airtight container, protected from light. |  |
| Shelf life | 1 years when properly stored |  |

Xiaoyou              linzi                         zhang shiming

Analys             Corrector                    Quality Assurance Officer